# Product Requirements Document (PRD)

NeuroLink — Educational Brain-Computer Interface Simulator

Version 1.0 | Python 3.12 | CE-SE-A | Team 2

Version: 1.0 | Runtime: Python 3.12 | Class: CE-SE-A | Team: 2

# 1. Executive summary

NeuroLink is a local educational web app demonstrating a simplified Brain-Computer Interface (BCI) signal-processing pipeline. It generates synthetic EEG-like waveforms, filters them, extracts frequency-domain features, classifies them into one of two illustrative labels, and visualizes the process in a dashboard. It is designed for a Biology for Engineers experiential-learning assignment.

Scientific boundary: The MVP uses generated signals, not human EEG. `left` and `right` are artificial demo labels, not evidence of a person's intention. This is not a medical device and must not be described as mind-reading or clinically validated.

# 2. Problem and goals

Real EEG hardware and data collection add cost and complexity to a classroom demonstration. A local simulator lets students explore signal processing without a headset.

Goals:

Demonstrate signal generation → filtering → feature extraction → classification → simulated output.

Show raw and filtered waveforms together.

Display named features and the predicted demo label.

Run locally on Python 3.12 with no paid API, cloud account, or credentials.

Include automated tests and accurate limitations.

# 3. Non-goals

Live EEG headset integration in the MVP.

Diagnosis, treatment, rehabilitation, or clinical claims.

Actual thought/intent decoding.

Production real-time BCI.

Comprehensive motion-artifact removal.

Cloud hosting, accounts, analytics, telemetry, or paid APIs.

Using synthetic accuracy as evidence of real-world BCI performance.

# 4. Users and stories

Primary users are undergraduate students presenting the project; secondary users are instructors and classroom audiences.

As a student, I can select a demo class and generate a waveform.

I can compare raw and filtered traces to explain filtering.

I can inspect feature values and understand how classification works.

I can see the prediction and model outputs.

I can run the project locally without hardware or credentials.

I receive actionable errors if the model is missing or inputs are invalid.

# 5. MVP requirements

## F-01 Synthetic signal generation (P0)

Generate a 2-second signal at 256 Hz for labels `left` and `right`, with noise and deliberately distinct frequency patterns. Support deterministic seeds and validate parameters. Default output must contain 512 samples and a matching time axis.

## F-02 Filtering (P0)

Apply a Butterworth band-pass filter from 1–40 Hz using second-order sections and zero-phase offline filtering. Preserve valid input shape and produce finite values. Explain that this does not remove all motion artifacts and is not a real-time streaming implementation.

## F-03 Feature extraction (P0)

Remove the mean, compute a real FFT, and calculate mean spectral power in 1–12 Hz and 12–40 Hz bands. Features must have stable names/order and finite values; empty bands must not silently yield NaN.

## F-04 Training (P0)

Create a reproducible balanced synthetic dataset. Use a stratified train/test split with fixed random state. Train a scikit-learn pipeline using `StandardScaler` and `LogisticRegression`; persist with joblib to a predictable project-relative path. Print held-out metrics labelled as synthetic-data-only.

## F-05 Dashboard (P0)

Build a Streamlit app titled “NeuroLink — BCI Simulator”. Let the user select `left` or `right` and click a button to generate/process a signal. Plot raw and filtered traces using Plotly; display named feature values, predicted demo command, and model output scores. Keep the disclaimer visible and show helpful errors.

## F-06 Learning content (P0)

Include “How it works” and limitations sections explaining synthetic data, arbitrary amplitude, interpretation of model outputs, and motion artifacts.

## F-07 Simulated cursor (P2, optional)

Only after P0 passes, optionally map `left` to decrement and `right` to increment a cursor position in [0,100]. Persist state across Streamlit reruns and label this as a simulation.

# 6. Technical requirements

Python 3.12; NumPy, SciPy, scikit-learn, joblib, Streamlit, Plotly, pytest.

Modular, testable code; useful type hints/docstrings.

Reproducible seeds for tests and training data.

Project-relative model paths.

Local-only operation; no data transmission or secrets.

`.gitignore` for venvs, caches, and generated artifacts.

Clear errors for invalid inputs and missing model.

README with setup, train, run, and test commands.

# 7. Signal specification

Sampling rate: 256 samples/second.

Default duration: 2 seconds / 512 samples.

Filter: Butterworth 1–40 Hz, SOS, zero-phase offline filtering.

Features: mean power for 1–12 Hz and 12–40 Hz.

Classifier: standardized logistic regression.

Labels: `left`, `right`, strictly artificial demo labels.

These are educational choices, not universal or clinical EEG settings.

# 8. UX requirements

Prominent “Educational prototype using simulated EEG signals — not live brain data” notice.

X-axis “Time (seconds)”; Y-axis “Amplitude (arbitrary units)”.

Feature names instead of an unlabeled array.

Raw and filtered traces easy to compare.

Do not imply that the selected class is the user's actual intention.

Do not rely on color alone to communicate information.

# 9. Risks and mitigations

| Risk | Mitigation |

|---|---|

| Synthetic data mistaken for real EEG | Persistent disclaimer and explicit labels |

| High synthetic accuracy overinterpreted | Label metrics synthetic-only |

| Filter claimed to remove all artifacts | Explain artifacts may overlap useful frequencies |

| Missing model causes crash | Actionable instruction to train first |

| Working directory breaks model loading | Use project-relative paths |

| Scope expands to hardware too early | Keep hardware and real datasets out of MVP |

# 10. Deliverables

Working local Streamlit app.

Modular generation, processing, feature, dataset, training, and inference code.

Automated tests.

Setup/run/test README.

Science and limitations guide.

Synthetic-only evaluation output.

Optional cursor demo if time permits.

Implementation notes containing actual tests, limitations, and deviations.

# 11. Definition of done

Fresh Python 3.12 environment installs dependencies.

Training saves a model and prints synthetic-only metrics.

App launches and processes both labels.

Raw/filtered plots, named features, prediction, and model outputs are visible.

Tests pass or failures are documented with evidence.

Disclaimers are clear.

README commands are accurate and implementation notes report actual verification.

# 12. Future work (not MVP)

A documented open EEG dataset, comparison of artifact-handling approaches, additional classifiers, or real-time hardware can be considered only as a separately scoped extension. If real data is added, cite its source/license, avoid data leakage, use held-out evaluation, and report limitations.